Digital System Design

Verilog Assignment

Semester III

UNIVERSITY OF DELHI

FACULTY OF TECHNOLOGY

Submitted To: Dr. Khushwant Sehra (Assistant Professor, ECE Department)

Submitted By: Manav Bhatia

Branch: Computer Science and Engineering (CSE-A)

Roll No. 24293916122

Enrollment No. 24DOCEBTCS000122

Ques. Writing Source, Testbench Codes and Running Simulations for the following:

- Basic Gates

- Boolean Expressions

- Multiplexers (4 x 1 and 8 x 1)

- Priority Encoder

- Decoder

- Half Adder and Full Adder

- Half Subtractor and Full Subtractor

- Universal Adder / Subtractor with Overflow Check

- SR Latch

- SR and JK - Flip Flop

- D and T - Flip Flop

- Counter Design

Ans.

- Basic Gates

- AND Gate

Source Code:

`timescale 1ns/1ps

module and_gate (

input  wire a,

input  wire b,

output wire y

);

assign y = a & b;

endmodule

Testbench Code:

`timescale 1ns/1ps

module tb_and_gate;

reg a, b;

wire y;

// Instantiate AND gate module

and_gate uut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- OR Gate

Source Code:

`timescale 1ns/1ps

module or_gate (

input  wire a,

input  wire b,

output wire y

);

assign y = a | b;

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_or_gate;

reg a, b;

wire y;

// Instantiate OR gate module

or_gate uut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- NOT Gate

Source Code:

`timescale 1ns/1ps

module not_gate (

input  wire a,

output wire y

);

assign y = ~a;

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_not_gate;

reg a;

wire y;

// Instantiate NOT gate module

not_gate uut (

.a(a),

.y(y)

);

initial begin

a = 0; #10;

a = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- NAND Gate

Source Code:

`timescale 1ns/1ps

module nand_gate (

input  wire a,

input  wire b,

output wire y

);

assign y = ~(a & b);

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_nand_gate;

reg a, b;

wire y;

// Instantiate NAND gate module

nand_gate uut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- NOR Gate

Source Code:

`timescale 1ns/1ps

module nor_gate (

input  wire a,

input  wire b,

output wire y

);

assign y = ~(a | b);

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_nor_gate;

reg a, b;

wire y;

// Instantiate NOR gate module

nor_gate uut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- XOR Gate

Source Code:

`timescale 1ns/1ps

module xor_gate (

input  wire a,

input  wire b,

output wire y

);

assign y = a ^ b;

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_xor_gate;

reg a, b;

wire y;

// Instantiate XOR gate module

xor_gate uut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- XNOR Gate

Source Code:

`timescale 1ns/1ps

module xnor_gate (

input  wire a,

input  wire b,

output wire y

);

assign y = ~(a ^ b);

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_xnor_gate;

reg a, b;

wire y;

// Instantiate XNOR gate module

xnor_gate uut (

.a(a),

.b(b),

.y(y)

);

initial begin

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Boolean Expressions

- Simple Behavioural

Source Code:

module bool_expr_beh(input a, input b, input c, output y);

assign y = (a & b) | (~c);

endmodule

Testbench Code:

`timescale 1ns/1ps

module tb_bool_expr_beh;

reg a, b, c;

wire y;

bool_expr_beh uut(

.a(a),

.b(b),

.c(c),

.y(y)

);

initial begin

a = 0; b = 0; c = 0; #10;

a = 0; b = 0; c = 1; #10;

a = 0; b = 1; c = 0; #10;

a = 0; b = 1; c = 1; #10;

a = 1; b = 0; c = 0; #10;

a = 1; b = 0; c = 1; #10;

a = 1; b = 1; c = 0; #10;

a = 1; b = 1; c = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Complex Behavioural

Source Code:

module bool_expr_complex_beh(input a, input b, input c, input d, output y);

assign y = (a & b) | (c ^ d) & (~a | b);

endmodule

Testbench Code:

`timescale 1ns/1ps

module tb_bool_expr_complex_beh;

reg a, b, c, d;

wire y;

bool_expr_complex_beh uut(

.a(a),

.b(b),

.c(c),

.d(d),

.y(y)

);

initial begin

a=0; b=0; c=0; d=0; #10;

a=0; b=0; c=0; d=1; #10;

a=0; b=0; c=1; d=0; #10;

a=0; b=0; c=1; d=1; #10;

a=0; b=1; c=0; d=0; #10;

a=0; b=1; c=0; d=1; #10;

a=0; b=1; c=1; d=0; #10;

a=0; b=1; c=1; d=1; #10;

a=1; b=0; c=0; d=0; #10;

a=1; b=0; c=0; d=1; #10;

a=1; b=0; c=1; d=0; #10;

a=1; b=0; c=1; d=1; #10;

a=1; b=1; c=0; d=0; #10;

a=1; b=1; c=0; d=1; #10;

a=1; b=1; c=1; d=0; #10;

a=1; b=1; c=1; d=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Simple Structural

Source Code:

`timescale 1ns / 1ps

module bool_expr_struct(input a, input b, input c, output y);

wire w1, w2;

and (w1, a, b);

not (w2, c);

or (y, w1, w2);

endmodule

Testbench Code:

`timescale 1ns/1ps

module tb_bool_expr_struct;

reg a, b, c;

wire y;

bool_expr_struct uut(

.a(a),

.b(b),

.c(c),

.y(y)

);

initial begin

a=0; b=0; c=0; #10;

a=0; b=0; c=1; #10;

a=0; b=1; c=0; #10;

a=0; b=1; c=1; #10;

a=1; b=0; c=0; #10;

a=1; b=0; c=1; #10;

a=1; b=1; c=0; #10;

a=1; b=1; c=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Complex Structural

Source Code:

module bool_expr_complex_struct(input a, input b, input c, input d, output y);

wire w1, w2, w3, w4, w5;

and (w1, a, b);

xor (w2, c, d);

or (w3, w1, w2);

not (w4, a);

or (w5, w4, b);

and (y, w5, w3);

endmodule

Testbench Code:

`timescale 1ns/1ps

module tb_bool_expr_complex_struct;

reg a, b, c, d;

wire y;

bool_expr_complex_struct uut(

.a(a),

.b(b),

.c(c),

.d(d),

.y(y)

);

initial begin

a=0; b=0; c=0; d=0; #10;

a=0; b=0; c=0; d=1; #10;

a=0; b=0; c=1; d=0; #10;

a=0; b=0; c=1; d=1; #10;

a=0; b=1; c=0; d=0; #10;

a=0; b=1; c=0; d=1; #10;

a=0; b=1; c=1; d=0; #10;

a=0; b=1; c=1; d=1; #10;

a=1; b=0; c=0; d=0; #10;

a=1; b=0; c=0; d=1; #10;

a=1; b=0; c=1; d=0; #10;

a=1; b=0; c=1; d=1; #10;

a=1; b=1; c=0; d=0; #10;

a=1; b=1; c=0; d=1; #10;

a=1; b=1; c=1; d=0; #10;

a=1; b=1; c=1; d=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Multiplexers (4 x 1 and 8 x 1)

- Mux 4x1

Source Code:

`timescale 1ns/1ps

module mux4to1 (

input wire [3:0] I,    // 4 data inputs: I[3:0]

input wire [1:0] S,    // 2 select lines: S[1:0]

output wire Y

);

assign Y = (S == 2'b00) ? I[0] :

(S == 2'b01) ? I[1] :

(S == 2'b10) ? I[2] :

I[3];

endmodule

Testbench Code:

module tb_mux4x1;

reg  [3:0] I;

reg  [1:0] S;

wire Y;

mux4x1 dut(I, S, Y);

initial begin

// Test all combinations

I = 4'b1010;  // arbitrary pattern

S = 2'b00; #10;

S = 2'b01; #10;

S = 2'b10; #10;

S = 2'b11; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Mux 8x1

Source Code:

`timescale 1ns / 1ps

module mux8to1(

input a, b, c, d, e, f, g, h,

input s1, s2, s3,

output reg y

);

always @(*) begin

if (s1 == 0 && s2 == 0 && s3 == 0) y = a;

else if (s1 == 0 && s2 == 0 && s3 == 1) y = b;

else if (s1 == 0 && s2 == 1 && s3 == 0) y = c;

else if (s1 == 0 && s2 == 1 && s3 == 1) y = d;

else if (s1 == 1 && s2 == 0 && s3 == 0) y = e;

else if (s1 == 1 && s2 == 0 && s3 == 1) y = f;

else if (s1 == 1 && s2 == 1 && s3 == 0) y = g;

else y = h;

end

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_mux8to1();

reg a, b, c, d, e, f, g, h;

reg s1, s2, s3;

wire y;

mux8to1 uut(

.a(a), .b(b), .c(c), .d(d), .e(e), .f(f), .g(g), .h(h),

.s1(s1), .s2(s2), .s3(s3),

.y(y)

);

initial begin

a = 1; b = 1; c = 1; d = 1;

e = 1; f = 1; g = 1; h = 1;

s1 = 0; s2 = 0; s3 = 0; #10;

s1 = 0; s2 = 0; s3 = 1; #10;

s1 = 0; s2 = 1; s3 = 0; #10;

s1 = 0; s2 = 1; s3 = 1; #10;

s1 = 1; s2 = 0; s3 = 0; #10;

s1 = 1; s2 = 0; s3 = 1; #10;

s1 = 1; s2 = 1; s3 = 0; #10;

s1 = 1; s2 = 1; s3 = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Priority Encoder

- 4 to 2 Priority Encoder

Source Code:

`timescale 1ns / 1ps

module priority_encoder(

input d0,d1,d2,d3,

output a,b,v

);

assign a = d2|d3;

assign b=( d1 &(~d2))|d3;

assign v = d3|d2|d1|d0;

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_priority_encoder(

);

reg d0,d1,d2,d3;

wire a,b ,v;

priority_encoder uut(d0,d1,d2,d3,a,b,v);

initial

begin

d0=0; d1=0;d2=0;d3=0;

#10

d0=1; d1=0;d2=0;d3=0;

#10

d0=0; d1=1;d2=0;d3=0;

#10

d0=0; d1=0;d2=1;d3=0;

#10

d0=0; d1=0;d2=0;d3=1;

#10

d0=1; d1=1;d2=0;d3=0;

#10

d0=1; d1=1;d2=1;d3=0;

#10

d0=1; d1=1;d2=1;d3=1;

#10

d0=1; d1=0;d2=1;d3=0;

#10

d0=1; d1=0;d2=0;d3=1;

#10

d0=1; d1=0;d2=1;d3=1;

#10

d0=0; d1=0;d2=1;d3=1;

#10

d0=0; d1=1;d2=1;d3=1;

#10

d0=1; d1=1;d2=0;d3=1;

#10

d0=0; d1=1;d2=1;d3=0;

#10

d0=0; d1=1;d2=0;d3=1;

#10

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Decoder (2 to 4)

Source Code:

`timescale 1ns / 1ps

module decoder(

input a,b,

output d0,d1,d2,d3

);

assign d0 = ~a&(~b);

assign d1 = ~a&b;

assign d2 = a&(~b);

assign d3 = a&b;

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_decoder(

);

reg a,b;

wire d0,d1,d2,d3;

decoder uut(a,b,d0,d1,d2,d3);

initial

begin

a =0; b=0;

#10

a =0; b=1;

#10

a =1; b=0;

#10

a =1; b=1;

#10

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Half Adder

Source Code:

`timescale 1ns / 1ps

module half_adder(

input a, b,

output sum, carry

);

assign sum = a ^ b;

assign carry = a & b;

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_half_adder;

reg a, b;

wire sum, carry;

half_adder dut(a, b, sum, carry);

initial begin

$display("A B | SUM CARRY");

a = 0; b = 0; #10;

a = 0; b = 1; #10;

a = 1; b = 0; #10;

a = 1; b = 1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Full Adder

Source Code:

`timescale 1ns / 1ps

module full_adder(

input a, b, cin,

output sum, cout

);

assign sum  = a ^ b ^ cin;

assign cout = (a & b) | (b & cin) | (a & cin);

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_full_adder;

reg a, b, cin;

wire sum, cout;

full_adder dut(a, b, cin, sum, cout);

initial begin

a=0; b=0; cin=0; #10;

a=1; b=0; cin=1; #10;

a=1; b=1; cin=0; #10;

a=1; b=1; cin=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Half Subtractor

Source Code:

`timescale 1ns / 1ps

module half_subtractor(

input a, b,

output diff, borrow

);

assign diff   = a ^ b;

assign borrow = (~a) & b;

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_half_subtractor;

reg a, b;

wire diff, borrow;

half_subtractor dut(a, b, diff, borrow);

initial begin

a=0; b=0; #10;

a=0; b=1; #10;

a=1; b=0; #10;

a=1; b=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Full Subtractor

Source Code:

`timescale 1ns / 1ps

module full_subtractor(

input a, b, bin,

output diff, bout

);

assign diff = a ^ b ^ bin;

assign bout = (~a & b) | (b & bin) | (~a & bin);

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_full_subtractor;

reg a, b, bin;

wire diff, bout;

full_subtractor dut(a, b, bin, diff, bout);

initial begin

a=0; b=0; bin=0; #10;

a=1; b=0; bin=1; #10;

a=1; b=1; bin=0; #10;

a=1; b=1; bin=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Universal Adder / Subtractor with Overflow Check

Source Code:

`timescale 1ns / 1ps

module univ_adder_subtractor(

input  [3:0] a, b,

input  mode,               // 0 = add, 1 = subtract

output [3:0] result,

output cout, overflow

);

wire [3:0] b_mod = mode ? ~b : b;

wire cin = mode;

assign {cout, result} = a + b_mod + cin;

assign overflow = (a[3] ~^ b_mod[3]) & (result[3] ^ a[3]);

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_univ_adder_subtractor;

reg [3:0] a, b;

reg mode;

wire [3:0] result;

wire cout, overflow;

univ_adder_subtractor dut(a, b, mode, result, cout, overflow);

initial begin

a=4'b0101; b=4'b0011; mode=0; #10;

a=4'b0111; b=4'b1000; mode=0; #10;

a=4'b0110; b=4'b0001; mode=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- SR Latch

Source Code:

`timescale 1ns / 1ps

module sr_latch(

input S, R,

output Q, Qbar

);

assign Q    = ~(R | Qbar);

assign Qbar = ~(S | Q);

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_sr_latch;

reg S, R;

wire Q, Qbar;

sr_latch dut(S, R, Q, Qbar);

initial begin

S=0; R=0; #10;

S=1; R=0; #10;

S=0; R=1; #10;

S=1; R=1; #10; // invalid

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- SR - Flip Flop

Source Code:

`timescale 1ns / 1ps

module sr_ff(

input S, R, clk,

output reg Q

);

always @(posedge clk) begin

if (S && ~R)      Q <= 1;

else if (R && ~S) Q <= 0;

else              Q <= Q;

end

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_sr_ff;

reg S, R, clk;

wire Q;

sr_ff dut(S, R, clk, Q);

initial clk = 0;

always #5 clk = ~clk;

initial begin

S=0; R=0; #10;

S=1; R=0; #10;

S=0; R=1; #10;

S=1; R=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- JK - Flip Flop

Source Code:

`timescale 1ns / 1ps

module jk_ff(

input J, K, clk,

output reg Q

);

always @(posedge clk) begin

case ({J, K})

2'b00: Q <= Q;

2'b01: Q <= 0;

2'b10: Q <= 1;

2'b11: Q <= ~Q;

endcase

end

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_jk_ff;

reg J, K, clk;

wire Q;

jk_ff dut(J, K, clk, Q);

initial clk=0;

always #5 clk = ~clk;

initial begin

J=0; K=0; #10;

J=1; K=0; #10;

J=0; K=1; #10;

J=1; K=1; #20;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- D Flip Flop

Source Code:

`timescale 1ns / 1ps

module d_ff(

input D, clk,

output reg Q

);

always @(posedge clk)

Q <= D;

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_d_ff;

reg D, clk;

wire Q;

d_ff dut(D, clk, Q);

initial clk = 0;

always #5 clk = ~clk;

initial begin

D=0; #10;

D=1; #10;

D=0; #10;

D=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- T Flip Flop

Source Code:

`timescale 1ns / 1ps

module t_ff(

input T, clk,

output reg Q

);

always @(posedge clk) begin

if (T) Q <= ~Q;

else   Q <= Q;

end

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_t_ff;

reg T, clk;

wire Q;

t_ff dut(T, clk, Q);

initial clk = 0;

always #5 clk = ~clk;

initial begin

T=0; #10;

T=1; #10;

$finish;

end

endmodule

Simulation:

RTL Synthesis:

- Counter Design

Source Code:

`timescale 1ns / 1ps

module counter(

input clk,

input reset,

output reg [2:0] count

);

always @(posedge clk or posedge reset) begin

if (reset)

count <= 3'b000;                         // Start at 0

else begin

case (count)

3'b000: count <= 3'b011;             // 0 → 3

3'b011: count <= 3'b101;             // 3 → 5

3'b101: count <= 3'b110;             // 5 → 6

3'b110: count <= 3'b000;             // 6 → 0 (loop)

default: count <= 3'b000;            // Lockout correction

endcase

end

end

endmodule

Testbench Code:

`timescale 1ns / 1ps

module tb_counter;

reg clk, reset;

wire [2:0] count;

counter dut(clk, reset, count);

initial clk = 0;

always #5 clk = ~clk;

initial begin

reset = 1; #10;

reset = 0;

// cycles - taken 200 ns

#200;

$finish;

end

endmodule

Simulation:

RTL Synthesis: